Lab 1: Configuring Static Routing

Part 1: IP Addressing Table

| Interface / Device | IP Address | Decimal Mask | Default Gateway |
| --- | --- | --- | --- |
| Branch-1 Fa0/0 | 192.168.10.1 | 255.255.255.224 | — |
| PC-A | 192.168.10.2 | 255.255.255.224 | 192.168.10.1 |
| PC-B | 192.168.10.3 | 255.255.255.224 | 192.168.10.1 |
| Branch-2 Fa0/0 | 192.168.10.33 | 255.255.255.224 | — |
| PC-D | 192.168.10.34 | 255.255.255.224 | 192.168.10.33 |
| PC-E | 192.168.10.35 | 255.255.255.224 | 192.168.10.33 |
| Branch-1 S0/1 | 192.168.10.65 | 255.255.255.252 | — |
| Head-Office S0/1 | 192.168.10.66 | 255.255.255.252 | — |
| Branch-2 S0/2 | 192.168.10.69 | 255.255.255.252 | — |
| Head-Office S0/2 | 192.168.10.70 | 255.255.255.252 | — |
| Head-Office S0/0 | 200.200.200.1 | 255.255.255.252 | — |
| ISP S0/0 | 200.200.200.2 | 255.255.255.252 | — |
| ISP Fa0/0 | 200.200.200.5 | 255.255.255.252 | — |
| Web Server | 200.200.200.6 | 255.255.255.252 | 200.200.200.5 |

Part 4(m): Summarization of all 192.168.10.x networks

Networks to summarize:

| Network | 4th octet (binary) |
| --- | --- |
| 192.168.10.0 /27 | 0000 0000 |
| 192.168.10.32 /27 | 0010 0000 |
| 192.168.10.64 /30 | 0100 0000 |
| 192.168.10.68 /30 | 0100 0100 |

Comparing the 4th octets bit-by-bit from the left, only the 1st bit (value 0) is common to all four networks; the 2nd bit already differs (0 vs 1). The first three octets (192.168.10) are identical across all four networks.

Common prefix length = 24 (first three octets) + 1 (first bit of 4th octet) = 25 bits.

Summary network:  192.168.10.0 / 25      Mask:  255.255.255.128

Verification: 192.168.10.0/25 covers addresses 192.168.10.0 – 192.168.10.127, which includes all four subnets (.0/27, .32/27, .64/30, .68/30).

Static summary route configured on ISP:

ISP(config)# ip route 192.168.10.0 255.255.255.128 200.200.200.1